PROPULSION SYSTEM INTERNSHIP

WEEK 3: COMPONENT SIZING AND VALIDATION

Engine Cycle Study : Design point analysis, Thrust calculation and Component sizing

DOCUMENT AND TRAINEE DETAILS

| Author | Eliudi Elphace Tonda |
| --- | --- |
| Date | 17th October 2026 |
| Purpose | Component sizing of the Week 2 cycle output, validation against published data, error analysis, and r hievision of the model where the checks showed it was needed |
| Role | Propulsion System Intern |
| Reference engine | CFM56-7B26 (Boeing 737NG turbofan) |

Problem statement (from Week 1)

“Using a station-by-station model, calculate the sea-level static take-off thrust and TSFC of the CFM56-7B26 (ISA, M0), estimate the fan, core and nozzle flow areas, and compare with published ICAO and EASA data.”

1. Summary of what this week found

This week I sized the main flow-path components from the Week 2 cycle and then tried to break my own work. The checking changed the Week 2 conclusions more than I expected, so this document records both the sizing and the corrections. The headline numbers are in Table 1; the short version is below.

| Quantity | Week 2 (as delivered) | Week 3 revised (R2) | Published / reference | Nature of the check |
| --- | --- | --- | --- | --- |
| Take-off thrust | 116,948 N | 116,988 N | 116,988 N (26,300 lbf) | Calibrated, not independent |
| Airflow | 348.2 kg/s (from fan diameter) | 341.1 kg/s (input) | 341.1 kg/s (752 lb/s), one source | Now an input |
| Take-off fuel flow | 1.141 kg/s (−6.6 %) | 1.221 kg/s | 1.221 kg/s (ICAO) | Calibrated |
| TSFC | 0.3443 lb/(lbf·h) | 0.3685 lb/(lbf·h) | 0.3685 (from ICAO fuel flow and thrust) | Calibrated |
| Turbine entry T4 | 1615 K | 1602.1 K | Not published | Lumped parameter |
| Fan tip diameter | 1.549 m (used as an input) | 1.537 m (−0.8 %) | 1.549 m (61.0 in) | Independent |

Table 1. Headline results at sea-level static take-off (ISA, M0). “Calibrated” means the model was tuned to that number, so agreement is not evidence of correctness.

Main findings

- The fan diameter is the one clean validation. Sizing the fan from the published airflow gives 1.537 m against 1.549 m published (−0.8 %). Across a plausible range of fan-face Mach number and hub-to-tip ratio the result spans −3.6 % to +3.8 %, inside the ±5 % target set in Week 1.

- Week 2’s 0.03 % thrust agreement was a calibration result, not a validation. T4 was adjusted until thrust matched. Its TSFC check used a reference (0.330 lb/(lbf·h)) that turns out to be a generic CFM56-family figure. Against the ICAO fuel flow, the Week 2 model under-predicts fuel burn by 6.6 %.

- I found three defects in Week 2: the hot-gas properties imply a gas constant of 310.8 J/(kg·K) instead of 287; the LPT mass-flow factor leaves a 0.27 % power-balance residual (the Week 1 limit was 0.1 %); and the proposed “non-circular” fan-sizing route is still circular.

- Fixing the gas properties moves the answer a lot. At fixed T4 the thrust falls by 4.2 %. Re-trimming to thrust then needs T4 near 1,696 K and over-predicts fuel flow by 10.0 %. Matching ICAO fuel flow as well needs component efficiencies about 0.039 above my textbook values. I treat that offset as a lumped correction for losses I have not modelled.

- Uncertainty is larger than the Week 2 precision suggested. Combining the one-at-a-time sensitivities in quadrature gives about ±3.3 % on thrust at fixed T4, ±6.8 % on TSFC, and ±75 K on the inferred T4. Areas set by an assumed Mach number move by roughly −15 % to +23 % for a ±0.05 Mach change.

- One input is still unresolved. My Week 1 notes give a take-off OPR of 27.2 and Week 2 used 32.6. I could not retrieve the ICAO pressure-ratio field to settle it, so the revised model uses 27.2 provisionally and the effect is quantified in Section 7.

2. Review of Week 1 and Week 2

Before sizing anything I compared the two earlier reports line by line. Table 2 lists where they disagree with each other or with published data. Table 3 scores Week 2 against the success criteria I set myself in Week 1.

2.1 Inconsistencies between the two reports

| Item | Week 1 | Week 2 | Finding and resolution |
| --- | --- | --- | --- |
| Take-off thrust | “27300 lbs” in the text, 26,300 lbf in the table | 116,988 N (26,300 lbf) | 27,300 lbf is the -7B27 rating. 26,300 lbf is correct for the -7B26. The Week 1 text was a slip; Week 2 is right. |
| Take-off OPR | 27.2 (take-off), 32.7–32.8 (cruise) | 32.6 used at sea-level static | Conflict. In the sources I found, 32.6 is quoted for the 7B26 as a maximum-climb figure, not take-off. Revised to 27.2 for the take-off point (provisional, see 7.1). |
| Airflow | 341 kg/s (752 lb/s) | 348.2 kg/s from fan geometry | The two differ by +2.1 %. Week 1’s own target was “within 1 %”, so this was a hidden fail. Revised to use the published value as the input. |
| Fan pressure ratio | 1.65 (textbook) | 1.60 | Changed in Week 2 without a recorded reason. I keep 1.60 but it is the most influential physical assumption (Section 7). |
| Turbine entry temperature | 1280–1300 °C (1553–1573 K) | 1615 K (1342 °C), calibrated | Week 2’s value is 40–60 K above the Week 1 range. R2 gives 1602.1 K. T4 is a lumped fitting parameter, not a measurement. |
| TSFC reference | ICAO fuel flow 1.221 kg/s to be used | 0.330 lb/(lbf·h) from planefyi.com | The planefyi page is a generic CFM56 page (it lists a 6.0 bypass ratio and 68.3 in fan, which are not 7B26 values). The Week 1 plan was better. ICAO-derived value used now. |
| Fuel heating value | 43.15 MJ/kg | 43.1 MJ/kg | Difference is 0.1 %. Negligible. |
| Project length | “9 weeks” and “four weeks” | – | 28 Sep to 26 Oct is four to five weeks. “9 weeks” was a typing error. |
| Cruise TSFC | – | Text quotes 0.555; code prints 0.6199 | The 1.78 % cruise agreement cannot be reproduced from the report. Cruise claims are withdrawn (Section 8). |

Table 2. Week 1 versus Week 2 consistency check.

2.2 Week 2 scored against the Week 1 success criteria

| Week 1 criterion | Week 2 claim | Week 3 finding |
| --- | --- | --- |
| Calibrated thrust within 1 % | 0.03 % error | True, but obtained by tuning T4 to the target. It shows the tuning worked, not that the model is right. |
| TSFC within ±10 % | +4.3 % vs 0.330 | Against the ICAO-derived value the error is −6.6 %. Still inside ±10 %, but the sign and size change with the reference. |
| Airflow within 1 % of data | Not reported | Not met: 348.2 kg/s is +2.1 % from the published figure. |
| Fan diameter within ±5 % | Not assessed | Week 2’s reverse route returns the input diameter by construction (see 5.3). Real check now done: −0.8 %. |
| Power-balance residual below 0.1 % | Zero “by construction” | Recomputing it from the station table gives −0.27 % on the LPT. Not met; corrected. |
| Hand check and limiting cases | Pass | Still valid. I re-ran the model and reproduced 335.86 N·s/kg and 116,947 N exactly. |
| Unit and dimension consistency | Not listed as a finding | The hot-gas cp, γ and R are mutually inconsistent (R = 310.8 J/(kg·K) implied). Found this week. |

Table 3. Scorecard of Week 2 against the Week 1 plan.

3. Published reference data used

Only public data is used. Table 4 lists each value, where it came from, and how far I trust it. I kept a note of where I could not confirm something because honesty about that matters more than a tidy table.

| Quantity | Value | Source | Confidence and caveat |
| --- | --- | --- | --- |
| Rated take-off thrust | 116.99 kN (26,300 lbf) | ICAO Engine Emissions Databank row for CFM56-7B26, as reproduced in a Hong Kong EPD appendix; Aircraft Commerce CFM56-7B guide | High. Two sources agree. |
| Take-off fuel flow | 1.221 kg/s | ICAO databank (same row). Climb-out 0.999, approach 0.338, idle 0.113 kg/s | High as certification data. Test conditions assumed standard (sea level, ISA, no bleed). |
| Bypass ratio | 5.1 | ICAO databank; Aircraft Commerce; university course notes | High. |
| Take-off airflow | 752 lb/s = 341.1 kg/s | Week 1 citation (Aerocontact) | Medium. I could not re-verify it this week. Treated as ±2 % in the error analysis. |
| Fan tip diameter | 61.0 in = 1.549 m | Aircraft Commerce; Purdue AAE page | High. |
| Core layout | 3-stage LPC, 9-stage HPC, 1-stage HPT, 4-stage LPT | Aircraft Commerce | High. |
| Overall pressure ratio | 32.6 (7B26); 27.2 at take-off per Week 1 | University course notes (32.6); Week 1 note citing ICAO (27.2) | Low. Reference condition of 32.6 is unclear and I could not retrieve the 27.2 source. |
| Exhaust gas temperature limits | 950 °C maximum, 920 °C indicated redline | Aircraft Commerce | High as a limit. The sensing station is not stated in my source. |

Table 4. Published data and confidence. The EASA type-certificate data sheet was consulted for the rating list but its detail was not re-read this week.

4. Method: how the model was revised

4.1 Three model versions

To keep track of what each correction does, I carry three versions through the report (Table 5). W2 is exactly the Week 2 code. R1 and R2 are the revised models.

| Model | Gas model | Airflow | OPR | Calibrated to | Result T4 |
| --- | --- | --- | --- | --- | --- |
| W2 | Constant cp (cp,t = 1244, γt = 1.333) | 348.2 kg/s (geometry) | 32.6 | Thrust, by T4 | 1615 K |
| R1 | Temperature-dependent cp, R = 287 | 341.1 kg/s (published) | 27.2 | Thrust, by T4 | 1,696 K |
| R2 | Temperature-dependent cp, R = 287 | 341.1 kg/s (published) | 27.2 | Thrust and ICAO fuel flow, by T4 and an efficiency offset | 1602.1 K |

Table 5. Model versions used in this report.

4.2 Variable-property gas model (independent calculation)

The Week 2 model uses one cp and one γ for the cold side and another pair for the hot side. That is a stated assumption (A3), but I had never tested it. For the independent calculation I wrote a second implementation, varcycle.py, in which enthalpy h(T) and the entropy function φ(T) are integrated from a temperature-dependent cp. Air uses a polynomial in θ = T/1000 K of the Walsh and Fletcher form; combustion gas adds a term proportional to f/(1+f). Compression and expansion then use φ(T₂) − φ(T₁) = R ln(PR), with R = 287 J/(kg·K) throughout, so the gas constant is no longer free to drift.

I checked the air polynomial against standard tabulated cp. It agrees to better than 0.2 % up to 1200 K, is 0.6 % low at 1400 K and 2.2 % low at 1600 K. That high-temperature error is an uncertainty on the new model too, and I note it in Section 8. All component efficiencies and pressure losses are identical to Week 2, so the only change between W2 and R1 is the gas model, the airflow and the OPR.

4.3 Calibration of R2

R1 matches thrust by adjusting T4 alone but then burns about 10.0 % too much fuel. Two published numbers (thrust and fuel flow) allow two unknowns to be solved. I chose T4 and a single common offset δη added to all five turbomachinery efficiencies (fan, booster, HPC, HPT, LPT). The system solved is:

F(T4, δη) × ṁ₀ = 116,988 N        and        f(T4, δη) × ṁ₀ / (1 + BPR) = 1.221 kg/s

The solution is T4 = 1602.1 K and δη = +0.0387, which puts the efficiencies at fan 0.919, booster 0.899, HPC 0.909, HPT 0.939 and LPT 0.949. The turbine values are at the optimistic end of what I would expect. The honest reading is that δη is absorbing losses and effects that the model leaves out (Section 8). It is not evidence that the engine really has those efficiencies.

Important consequence: once thrust and fuel flow are calibrated, they can no longer validate the model. The independent checks that remain are the fan diameter, the plausibility of the temperatures against exhaust gas temperature limits, and stage-loading sanity checks.

5. Component sizing

5.1 Method and assumptions

Flow areas come from continuity using the compressible flow function, with γ evaluated from the local cp at the static temperature:

ṁ = Pt · A · √(γ / (R·Tt)) · M · [1 + (γ − 1)M²/2] ^ ( −(γ + 1) / (2(γ − 1)) )

Every area is calculated twice: once with this formula and once from ρ·V·A using static conditions. The two agree to four decimal places at every station, which checks the code but not the physics. The assumed Mach numbers (Table 6) are standard design-practice values, not published engine data, so they are the main uncertainty in the sizing.

| Station | Assumed Mach | Basis |
| --- | --- | --- |
| 2 Fan face | 0.60 | Carried from Week 2. Hub-to-tip ratio 0.30 (typical, not published for this engine). |
| 25 HPC inlet | 0.50 | Typical design value for a high-pressure compressor inlet. |
| 3 HPC exit | 0.25 | Typical low exit Mach number to limit diffuser losses. |
| Combustor reference | 25 m/s | Reference velocity of the order 20–30 m/s, based on the full HPC exit flow. |
| 4 HPT nozzle guide vane | 1.0 (choked) | Assumed choked at take-off. Flow excludes the cooling air bled at the HPC exit (Week 2 model). |
| 5 LPT exit | 0.45 | Typical design value. |
| 8 Core nozzle throat | 1.0 (choked) | Computed Pt9/P0 = 2.37, above the critical ratio of about 1.85, so choked. |
| 19 Bypass nozzle exit | 0.82 (computed) | Computed Pt19/P0 = 1.56, below the critical ratio, so not choked. |

Table 6. Sizing assumptions.

5.2 Sizing results for the revised model (R2)

| Station | Flow (kg/s) | Tt (K) | Pt (kPa) | Mach | Area (m²) | Equivalent circle diameter (m) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 Fan face (annulus) | 341.1 | 288.1 | 100.8 | 0.60 | 1.688 | 1.466 (tip 1.537) |
| 25 HPC inlet | 55.9 | 393.7 | 274.2 | 0.50 | 0.1342 | 0.413 |
| 3 HPC exit | 55.9 | 776.2 | 2742.3 | 0.25 | 0.0343 | 0.209 |
| Combustor reference | 55.9 | 776.2 | 2742.3 | 25 m/s | 0.1817 | 0.481 |
| 4 HPT NGV throat | 50.4 | 1602.1 | 2605.1 | 1.00 | 0.0197 | 0.158 |
| 5 LPT exit | 57.1 | 913.8 | 244.7 | 0.45 | 0.2588 | 0.574 |
| 8 Core nozzle throat | 57.1 | 913.8 | 239.8 | 1.00 | 0.1809 | 0.480 |
| 19 Bypass nozzle exit | 285.2 | 333.2 | 158.1 | 0.82 | 0.8385 | 1.033 |

Table 7. Flow areas at sea-level static take-off, model R2 (airflow 341.1 kg/s, core flow 55.9 kg/s, bypass flow 285.2 kg/s). “Equivalent circle diameter” is the diameter of a circle of the same area, with no hub, and is only a scale indicator.

5.3 Fan diameter: the independent validation

The fan annulus area is A = ṁ₀ / (ρ₂V₂) at the assumed face Mach number, and the tip diameter follows from D = √(4A / (π(1 − (r_hub/r_tip)²))). With the published airflow, M₂ = 0.60 and a hub-to-tip ratio of 0.30, I get 1.537 m against 1.549 m published (−0.8 %). This is a real check because neither the airflow nor the diameter was tuned to produce it.

Table 8 shows how much the answer depends on the two assumed numbers. The span of −3.6 % to +3.8 % stays inside ±5 %. Put the other way round, a face Mach number of 0.584 would close the gap exactly, which is within the normal design range.

| Fan-face Mach | Hub/tip 0.28 | Hub/tip 0.30 | Hub/tip 0.35 |
| --- | --- | --- | --- |
| 0.55 | 1.569 m (+1.3 %) | 1.579 m (+1.9 %) | 1.608 m (+3.8 %) |
| 0.60 | 1.527 m (−1.4 %) | 1.537 m (−0.8 %) | 1.565 m (+1.0 %) |
| 0.65 | 1.493 m (−3.6 %) | 1.502 m (−3.0 %) | 1.530 m (−1.3 %) |

Table 8. Fan tip diameter from the published airflow (341.1 kg/s) for a range of assumptions, with deviation from 1.549 m.

Why the Week 2 “non-circular route” does not work. Week 2 proposed deriving the airflow as thrust divided by the calibrated specific thrust and then sizing the fan from it. But T4 was tuned so that specific thrust × 348.2 kg/s equals the thrust, so thrust ÷ specific thrust returns 348.2 kg/s again, and 348.2 kg/s came from the published diameter. The fan diameter then comes back at 1.553 m (+0.22 %), which looks excellent and means nothing. The published airflow is the only genuinely independent anchor, which is why I switched to it.

5.4 How much the cycle choices change the areas

| Area (m²) | W2 | R1 | R2 | W2 vs R2 |
| --- | --- | --- | --- | --- |
| Fan face (annulus) | 1.7235 | 1.6878 | 1.6878 | +2.1 % |
| HPC inlet | 0.1378 | 0.1351 | 0.1342 | +2.7 % |
| HPC exit | 0.0305 | 0.0349 | 0.0343 | −11.0 % |
| Combustor reference | 0.1740 | 0.1877 | 0.1817 | −4.2 % |
| HPT NGV throat | 0.0166 | 0.0203 | 0.0197 | −15.5 % |
| LPT exit | 0.2839 | 0.2922 | 0.2588 | +9.7 % |
| Core nozzle throat | 0.1991 | 0.2042 | 0.1809 | +10.0 % |
| Bypass nozzle exit | 0.8587 | 0.8410 | 0.8385 | +2.4 % |

Table 9. Flow areas from the three model versions. The spread shows how much the Week 2 errors would have carried into the sizing.

The hot-end areas (HPT throat, LPT exit, core nozzle) differ by roughly 10–16 % between W2 and R2. That is the same order as the model-form error and is the practical cost of the inconsistent hot-gas properties. The bypass nozzle and fan face move by only about 2 % because they sit on the cold side and depend mainly on airflow.

5.5 Sanity checks where no published component data exist

Apart from the fan diameter, I could not find published flow areas for this engine, so the rest of the sizing cannot be validated against data. What I can do is check that the numbers are physically reasonable and consistent with the published stage counts:

- HPC: the revised OPR of 27.2 with FPR 1.60 and booster 1.70 leaves an HPC pressure ratio of 10.0. Over nine stages that is 1.29 per stage and about 42.5 K temperature rise per stage, which is moderate for a high-pressure compressor.

- Booster: 1.70 over three stages is 1.19 per stage, also moderate.

- HPT: a single stage extracts 449 kJ/kg. For a stage loading of 2 to 3 (a rule of thumb for a highly loaded single stage) the mean blade speed would need to be about 387 to 474 m/s. That is at the upper end of what cooled turbine blades can take. This is a plausibility flag that T4 or the HPT work may be biased high; it is not a finding.

- LPT: four stages share 333 kJ/kg, about 83 kJ/kg per stage. Stage loadings of 1.5 to 2.5 would need mean blade speeds of 183 to 236 m/s, which is believable for a low-speed spool.

- HPC exit blade height (illustrative): with the assumed M = 0.25 the exit flow area is 0.0343 m². For a mean radius of 0.30 m (my assumption, not a published figure) that is a passage height of about 18 mm, which is the short-blade regime expected at the back of a high-OPR compressor.

The stage-loading figures use rules of thumb from general turbomachinery practice, not figures taken from a specific reference for this engine, and I flag them as indicative only.

6. Validation against published data

6.1 Comparison

Figure 1. Deviation from published values for the three model versions. R2 shows zero for fuel flow and TSFC because it was calibrated to them.

| Quantity | Published | W2 | R1 | R2 | Independent? |
| --- | --- | --- | --- | --- | --- |
| Thrust (N) | 116,988 | 116,948 (−0.03 %) | 116,988 | 116,988 | No, calibrated |
| Airflow (kg/s) | 341.1 | 348.2 (+2.1 %) | 341.1 (input) | 341.1 (input) | Input in R1, R2 |
| Fuel flow (kg/s) | 1.221 | 1.141 (−6.6 %) | 1.343 (+10.0 %) | 1.221 | No in R2; yes in W2, R1 |
| TSFC (lb/(lbf·h)) | 0.3685 (derived) | 0.3443 (−6.6 %) | 0.4052 (+10.0 %) | 0.3685 | Same as fuel flow |
| Fan tip diameter (m) | 1.549 | 1.553 (circular) | 1.537 (−0.8 %) | 1.537 (−0.8 %) | Yes |
| Bypass ratio | 5.1 | 5.1 | 5.1 | 5.1 | Input |
| Exhaust temperature check | ≤ 950 °C limit (920 °C indicated) | HPT exit 882 °C | HPT exit 994 °C | HPT exit 920 °C | Plausibility only |

Table 10. Model results against published data.

6.2 Interpretation

Against the ICAO fuel flow, the model I handed in at Week 2 under-predicted fuel burn by 6.6 %. After I corrected the gas properties (R1) it over-predicted by 10.0 %. Both are within the ±10 % target from Week 1, but only just for R1, and neither error is small. R2 closes the gap by construction, so its fuel-flow agreement carries no information.

The model’s HPT-exit total temperature in R2 is 920 °C. The published limits are 950 °C maximum and 920 °C indicated for take-off. The real sensor sits downstream of the HPT, so the true reading for my model would be lower and I cannot draw a firm conclusion. Still, a healthy engine should sit comfortably below its redline, so I read this as a mild hint that the calibrated T4 is a little high, consistent with the HPT-loading flag in 5.5.

6.3 Independent calculation: why agreement can mislead

There is a trap in these numbers. If I keep the Week 2 gas model, re-anchor to the published airflow and use OPR 27.2, the re-trimmed T4 is 1635 K and the fuel flow comes out at 1.219 kg/s, only −0.2 % from ICAO. That looks like an excellent validation. It is not. If I instead keep the constant-property approach but make the gas constant consistent (cp = 1149 J/(kg·K) with γ = 1.333, or cp = 1200 with γ = 1.315), the fuel flow becomes +5.0 % and +6.4 % off. The near-perfect match came from two errors cancelling.

Table 11 compares the two gas models at the same T4. For the same inputs the constant-property model over-predicts specific thrust by about 3.5–4 %, because a hot-gas cp of 1244 with R = 287 gives the turbine too much expansion headroom and the nozzle too much velocity.

| Case (same T4, same efficiencies) | Spec. thrust, constant cp (N·s/kg) | Spec. thrust, variable cp (N·s/kg) | Difference | TSFC var. cp |
| --- | --- | --- | --- | --- |
| T4 = 1615 K, OPR 32.6 | 335.86 | 321.84 | −4.2 % | 0.3693 |
| T4 = 1635 K, OPR 27.2 | 342.97 | 330.98 | −3.5 % | 0.3878 |

Table 11. Gas-model comparison at fixed inputs.

7. Errors and uncertainty

7.1 Sources of error and their size

| Source | Type | Size | Effect on conclusions |
| --- | --- | --- | --- |
| Published airflow (single source) | Data | ±2 % tested | T4 shifts by −37 to +41 K and TSFC by +3.5 % to −3.1 %. Fan diameter moves by about ±1 %. |
| Take-off OPR (27.2 vs 32.6) | Data / definition | Full recalibration at 32.6 gives T4 1641.1 K and δη +0.0286 | Thrust and fuel flow still match, so they cannot choose between the two. Turbine and compressor outlet temperatures change, so the hot-end and HPC-exit areas depend on this unresolved choice. |
| Gas-property model | Model form | Specific thrust −3.5 % to −4.2 % at fixed T4 | Raises inferred T4 by about 60–110 K. Largest single model-form error found. |
| Component efficiencies and losses | Assumption | ±0.02 on each efficiency; loss ranges in Table 12 | Each moves trimmed TSFC by about ±1 %. Nozzle pressure ratio is the largest at about ±2.4 %. |
| Fan pressure ratio | Assumption | 1.55 to 1.65 | TSFC −3.0 % / +4.1 %. The most influential physical input, and Week 1 and Week 2 disagree on it. |
| Cooling air fraction | Assumption | 0.08 to 0.16 | T4 shifts by about ∓40 K. Strongly coupled to T4 and δη. |
| Mach number and hub-to-tip ratio in sizing | Assumption | ±0.05 Mach; hub/tip 0.28–0.35 | Fan diameter −3.6 % to +3.8 %. Area at HPC exit −15 % to +23 %, HPC inlet −6 % to +8 %, LPT exit −8 % to +10 %. |
| Air polynomial at high temperature | Model form | −0.6 % at 1400 K, −2.2 % at 1600 K | Probably a small contribution to the hot-end temperature rise. Not separately propagated. |
| Numerical solution | Numerical | Residuals below 1e-6 of the target | Negligible compared with everything above. |

Table 12. Error budget. Sizes for the assumptions come from the one-at-a-time study in 7.2.

7.2 One-at-a-time sensitivity of the revised model

As planned in Week 1, I changed each uncertain input by a plausible amount while holding the others at their R2 values. Three outputs are reported. The first is the change in thrust at fixed T4 and airflow, which shows how wrong an uncalibrated model could be. The second is the T4 change needed to restore thrust. The third is the change in TSFC after that re-trim, which is the figure that matters when T4 is used as the tuning knob. The efficiency offset δη is held at +0.0387 during this study.

| Parameter (low – high) | Δ thrust at fixed T4 (%) | Δ T4 to restore thrust (K) | Δ TSFC after re-trim (%) |
| --- | --- | --- | --- |
| Fan efficiency (±0.02) | −0.49 / +0.46 | +9 / −9 | +1.04 / −0.99 |
| Booster efficiency (±0.02) | −0.22 / +0.21 | +4 / −4 | +0.29 / −0.28 |
| HPC efficiency (±0.02) | −0.76 / +0.70 | +14 / −14 | +1.03 / −0.98 |
| HPT efficiency (±0.02) | −0.48 / +0.44 | +9 / −9 | +1.24 / −1.17 |
| LPT efficiency (±0.02) | −0.47 / +0.43 | +9 / −9 | +1.22 / −1.16 |
| Burner pressure ratio (0.94–0.96) | −0.17 / +0.16 | +3 / −3 | +0.44 / −0.43 |
| Nozzle pressure ratio (0.97–0.99) | −0.91 / +0.89 | +18 / −17 | +2.43 / −2.31 |
| Inlet recovery (0.990–1.000) | −0.45 / +0.44 | +9 / −8 | +1.18 / −1.15 |
| Cooling fraction (0.08–0.16) | +2.10 / −2.36 | −41 / +45 | −1.26 / +1.33 |
| Burner efficiency (0.98–1.00) | +0.03 / −0.03 | −1 / +1 | +1.03 / −1.01 |
| Mechanical efficiency (0.985–0.995) | −0.31 / +0.30 | +6 / −6 | +0.80 / −0.79 |
| Fan pressure ratio (1.55–1.65) | −1.42 / +1.23 | +30 / −22 | +4.08 / −3.01 |
| Booster pressure ratio (1.6–1.8) | +0.01 / −0.01 | −0 / +0 | +0.03 / −0.03 |
| Bypass ratio (5.0–5.2) | +0.65 / −0.63 | −12 / +12 | −0.05 / +0.06 |
| Take-off OPR (25.2–29.2) | +0.26 / −0.29 | −5 / +5 | +1.15 / −1.00 |
| Published airflow (±2 %) | +0.00 / +0.00 | +41 / −37 | +3.49 / −3.05 |

Table 13. Sensitivity results. Each cell shows the result for the low value, then the high value, of the range in the first column.

Figure 2. Sensitivity of TSFC (after re-trimming T4 to hold thrust) to each uncertain input.

7.3 Combined uncertainty and what it means

Taking the larger side of each range and combining them in quadrature gives about ±3.3 % on thrust at fixed T4, ±6.8 % on TSFC after re-trim, and ±75 K on the inferred T4. This treats the inputs as independent, which overstates the benefit of cancellation; in reality efficiencies, cooling fraction and T4 are strongly coupled. These numbers also do not include the model-form errors in Table 12, which are of similar size.

What this means for the conclusions:

- A thrust match of 0.03 % is far smaller than the ±3 % this method can honestly predict from its inputs. It should be described as a calibration result.

- A TSFC agreement of about ±10 % is the most this approach can claim, which matches the target I set in Week 1 and no tighter.

- The fan-diameter check is robust because it depends only on airflow, a Mach number and a hub ratio, none of which pass through the uncertain cycle model.

- The inferred T4 (about 1600 K) should not be quoted as the engine’s real turbine entry temperature. Its uncertainty is of order ±75 K plus the model-form offset.

8. Where the approach could fail

These are the weaknesses I am aware of, in rough order of importance.

- T4 and δη cannot be separated. Two published numbers fix two unknowns, but several physical errors (cooling model, nozzle losses, efficiencies, OPR) produce the same symptom. R2 hides all of them inside two parameters.

- The OPR reference condition is unresolved. If the true take-off OPR is 32.6 the model still matches thrust and fuel flow but with different internal temperatures, which would change several of the sized areas.

- Single operating point. Everything is calibrated at sea-level static. Nothing here validates cruise or climb, and the Week 2 cruise TSFC claim is withdrawn because I cannot reproduce it. A fair cruise check would compare against the maximum-climb thrust, but needs a cruise airflow I do not have.

- One-dimensional flow. No inlet distortion, tip clearance, Reynolds effects or nozzle discharge and velocity coefficients. Nozzle losses are represented only by a total-pressure ratio of 0.98.

- Crude cooling model. Cooling air is removed at the HPC exit, does no work, and re-enters after the HPT at HPC-exit temperature. Real engines inject it through the vanes and blades. This probably penalises the model and is partly interchangeable with δη.

- Sizing uses design-practice Mach numbers. Areas are flow areas only. They are not blade heights, casing diameters or mechanical designs, and no published component dimensions exist to check them against.

- Published data limits. The airflow has one source, the OPR is unclear, and the ICAO values are for a specific engine build and test set-up. Tolerances on these numbers are not stated.

- Polynomial gas model. Coefficients were checked against standard tables but not against a full thermochemical solver, and the polynomial is low by about 2 % at 1600 K.

9. Revisions made and revision log

Table 14 records every change I made after checking, the check that triggered it, and its effect. Anything I did not change is listed afterwards, with the reason.

| # | Change | Found by | Effect |
| --- | --- | --- | --- |
| C1 | Airflow anchor changed from 348.2 kg/s (fan geometry) to 341.1 kg/s (published) | Circularity check; Week 1 airflow criterion | Fan diameter is now an independent output. With the Week 2 gas model alone, T4 rises from 1615.6 to 1654.5 K. |
| C2 | LPT mass-flow factor corrected from (1 + f_burner) to (1 + f_total) | Power-balance residual recomputed from the station table | Residual −0.27 % → 0.00 %. Specific thrust 335.59 vs 335.86 N·s/kg (−0.08 %). The 0.9 K difference in Week 2’s hand check of Tt5 was this factor, not rounding. |
| C3 | Gas model replaced by temperature-dependent cp with R = 287 J/(kg·K) | Unit and consistency check (implied R = 310.8) | Specific thrust −3.5 % to −4.2 % at fixed T4; inferred T4 up by about 60–110 K. |
| C4 | Take-off OPR changed from 32.6 to 27.2 (provisional) | Week 1 versus Week 2 comparison | Inferred T4 down by about 17 K in the variable-property model; fuel flow down about 2 %. |
| C5 | TSFC reference changed from 0.330 to ICAO-derived 0.3685 | Source check on the 0.330 figure | Week 2 deviation changes from +4.3 % to −6.6 %. |
| C6 | Two-parameter calibration (T4 and δη = +0.0387) to thrust and ICAO fuel flow | R1 over-predicted fuel flow by 10.0 % | Thrust and fuel flow now match; no longer independent evidence. T4 = 1602.1 K. |
| C7 | Cruise TSFC claim withdrawn | Text quotes 0.555 but code prints 0.6199 | Cruise is declared unvalidated. |
| C8 | Sensitivity study completed as planned in Week 1 | Week 1 validation plan | Tables 12 and 13, Figure 2. |

Table 14. Revision log (continues the log required by the Week 1 documentation standard).

Left unchanged, deliberately: FPR 1.60 and booster ratio 1.70 (no better evidence to prefer 1.65, but the sensitivity is reported); cooling fraction 0.12 (coupled to T4 and δη, so changing it would not reveal anything new); hub-to-tip ratio 0.30 and fan-face Mach 0.60 (already tested over a range); the fuel heating value of 43.1 MJ/kg (the 0.1 % difference from Week 1 is negligible).

10. Conclusions and next steps

The sizing worked, and the fan diameter came out within 0.8 % of the published value without being tuned to it. The more valuable result is that checking exposed how much of the Week 2 agreement was calibration or compensating error. I now trust the cycle model to about ±10 % on TSFC and about ±3 % on thrust at fixed inputs, not the 0.03 % that Week 2 suggested.

For Week 4, the dossier and presentation:

- Confirm the take-off OPR from the ICAO databank pressure-ratio field, and find a second source for the 752 lb/s airflow.

- Replace the nozzle total-pressure ratio with velocity and discharge coefficients, and model cooling air re-injection through the turbine, so that δη has less to absorb.

- Cross-check the revised cycle against pyCycle or a textbook worked example if either is available, since Week 1 planned an industry-style independent check.

- Attempt a cruise validation against the published maximum-climb thrust once a defensible cruise airflow is available.

- Present the headline results with the uncertainty bounds from Section 7, labelling every calibrated number as calibrated.

Appendix A. Equations used in the revised model

- Gas model: cp,air(T) = 1000 Σ Aᵢ θⁱ J/(kg·K) for i = 0…8, θ = T/1000 K; combustion gas adds f/(1+f) · 1000 Σ Bᵢ θⁱ. Enthalpy h = ∫cp dT and entropy function φ = ∫(cp/T) dT, both integrated analytically from a 298.15 K reference.

- Compression or expansion: find T₂ₛ from φ(T₂ₛ) − φ(T₁) = R ln(PR); then h₂ = h₁ + (h₂ₛ − h₁)/η for a compressor, or h₂ = h₁ − (h₁ − h₂ₛ)η for a turbine.

- Burner fuel-air ratio: h_air(Tt3) + f·η_b·LHV = (1 + f)·h_gas(Tt4, f), solved numerically. LHV = 43.1 MJ/kg.

- Turbine work balance: HPT: (1 − c)(1 + f_b)·Δh_HPT·η_m = h(Tt3) − h(Tt25). Cooling air (fraction c) re-mixes after the HPT at Tt3 by energy balance. LPT: (1 + f_total)·Δh_LPT·η_m = (1 + BPR)·Δh_fan + Δh_booster.

- Nozzles: throat found from h(Tt) − h(T) = ½γRT; if the throat static pressure exceeds ambient the nozzle is choked and V_eff = V + (RT/V)(1 − P₀/P). Otherwise the exit is expanded to ambient.

- Specific thrust and TSFC: F/ṁ₀ = [(1 + f)V_eff,9 + BPR·V_eff,19] / (1 + BPR) at V₀ = 0; TSFC = f / ((1 + BPR) · F/ṁ₀), converted to lb/(lbf·h) by multiplying by 3600 × 2.20462 / 0.224809.

Appendix B. Code for the revised model and sizing

The three scripts below reproduce every number in Sections 4 to 7 when run with Python 3, NumPy and SciPy. The sensitivity study repeats the calibration call with one input changed at a time. The Week 2 files (cycle.py, mass_flow.py) are unchanged and are not repeated here.

varcycle.py

# variable-property cycle

"""Independent variable-property (T-dependent cp) check of the SLS take-off cycle.

cp_air(T) polynomial (Walsh & Fletcher form), combustion-gas correction proportional to FAR.

Same component efficiencies/losses as cycle.py so the ONLY change is the gas model."""

import numpy as np

from scipy.optimize import brentq

A=[0.992313,0.236688,-1.852148,6.083152,-8.893933,7.097112,-3.234725,0.794571,-0.081873]

B=[-0.718874,8.747481,-15.863157,17.254096,-10.233795,3.081778,-0.361112,-0.003919]

R=287.0; TREF=298.15; LHV=43.1e6

def _hp(c,th): return sum(c[i]*th**(i+1)/(i+1) for i in range(len(c)))

def _sp(c,th): return c[0]*np.log(th)+sum(c[i]*th**i/i for i in range(1,len(c)))

def cp(T,f=0.0):

th=T/1000.0

return 1000*(sum(A[i]*th**i for i in range(len(A)))+ f/(1+f)*sum(B[i]*th**i for i in range(len(B))))

def h(T,f=0.0):   # J/kg relative to TREF

th,tr=T/1000.0,TREF/1000.0

ha=1e6*(_hp(A,th)-_hp(A,tr)); hb=1e6*(_hp(B,th)-_hp(B,tr))

return ha+f/(1+f)*hb

def phi(T,f=0.0):

th=T/1000.0

return 1000*(_sp(A,th)+f/(1+f)*_sp(B,th))

def T_from_h(hv,f=0.0): return brentq(lambda T:h(T,f)-hv,150,3000)

def comp(Tin,PR,eta,f=0.0):

Ts=brentq(lambda T:phi(T,f)-phi(Tin,f)-R*np.log(PR),Tin,3000)

hout=h(Tin,f)+(h(Ts,f)-h(Tin,f))/eta

return T_from_h(hout,f)

def turb_drop(Tin,dh,eta,f,Pin):

hs=h(Tin,f)-dh/eta

Ts=T_from_h(hs,f); Pout=Pin*np.exp((phi(Ts,f)-phi(Tin,f))/R)

Tout=T_from_h(h(Tin,f)-dh,f); return Tout,Pout

def nozzle(Pt,Tt,P0,f):

# throat (M=1)

g=lambda T: cp(T,f)/(cp(T,f)-R)

Tth=brentq(lambda T:h(Tt,f)-h(T,f)-0.5*g(T)*R*T,200,Tt)

Pth=Pt*np.exp((phi(Tth,f)-phi(Tt,f))/R)

if Pth>P0:

V=np.sqrt(g(Tth)*R*Tth); return V+(R*Tth/V)*(1-P0/Pth),True,Tth,Pth

Te=brentq(lambda T:phi(T,f)-phi(Tt,f)-R*np.log(P0/Pt),200,Tt)

return np.sqrt(2*(h(Tt,f)-h(Te,f))),False,Te,P0

def cycle(T4,OPR=27.2,BPR=5.1,FPR=1.60,BPRb=1.70,cool=0.12,eta_f=0.88,eta_b=0.86,eta_c=0.87,eta_tH=0.90,eta_tL=0.91,

eta_burn=0.99,eta_m=0.99,pi_d=0.995,pi_b=0.95,pi_n=0.98,P0=101325.0,T0=288.15):

Tt2=T0;Pt2=P0*pi_d

Tt13=comp(Tt2,FPR,eta_f);Pt13=Pt2*FPR

Tt25=comp(Tt13,BPRb,eta_b);Pt25=Pt13*BPRb

HPC=OPR/(FPR*BPRb);Tt3=comp(Tt25,HPC,eta_c);Pt3=Pt25*HPC;Pt4=Pt3*pi_b

# fuel-air (burner) : h_a(Tt3)+f*eta*LHV=(1+f)*h_g(T4,f)

fb=brentq(lambda f:h(Tt3,0)+f*eta_burn*LHV-(1+f)*h(T4,f),0.001,0.06)

mb=1-cool; ftot=fb*mb

whpc=h(Tt3)-h(Tt25)

dh_hpt=(whpc/eta_m)/(mb*(1+fb))

Tt45g,Pt45=turb_drop(T4,dh_hpt,eta_tH,fb,Pt4)

hmix=(mb*(1+fb)*h(Tt45g,fb)+cool*h(Tt3,0))/(mb*(1+fb)+cool)

# composition of mixed stream

Tt45=T_from_h(hmix,ftot)

wlp=((1+BPR)*(h(Tt13)-h(Tt2))+(h(Tt25)-h(Tt13)))/eta_m

dh_lpt=wlp/(1+ftot)

Tt5,Pt5=turb_drop(Tt45,dh_lpt,eta_tL,ftot,Pt4*0+Pt45*1.0)

Ve9,ch9,T9,P9=nozzle(Pt5*pi_n,Tt5,P0,ftot)

Ve19,ch19,T19,P19=nozzle(Pt13*pi_n,Tt13,P0,0.0)

Fsp=((1+ftot)*Ve9+BPR*Ve19)/(1+BPR)

TSFC=ftot/((1+BPR)*Fsp)*2.20462*3600/0.224809

return dict(Tt2=Tt2,Pt2=Pt2,Tt13=Tt13,Pt13=Pt13,Tt25=Tt25,Pt25=Pt25,Tt3=Tt3,Pt3=Pt3,Tt4=T4,Pt4=Pt4,Tt45=Tt45,Pt45=Pt45,

Tt5=Tt5,Pt5=Pt5,Pt9=Pt5*pi_n,Pt19=Pt13*pi_n,T9=T9,P9=P9,T19=T19,Tt45g=Tt45g,

f=ftot,fb=fb,Fsp=Fsp,TSFC=TSFC,ch9=ch9,ch19=ch19,Ve9=Ve9,Ve19=Ve19,cool=cool,BPR=BPR,HPC_PR=HPC)

if __name__=="__main__":

import cycle as C

for (T4,OPR) in ((1615,32.6),(1635,27.2)):

v=cycle(T4,OPR=OPR); p=dict(C.BASE);p['OPR']=OPR;p.update(C.REF)

c=C.cycle((0,0),T4=T4,**p)

print(T4,OPR,'VAR Fsp %.2f TSFC %.4f f %.4f Tt3 %.1f Tt45 %.1f Tt5 %.1f'%(v['Fsp'],v['TSFC'],v['f'],v['Tt3'],v['Tt45'],v['Tt5']),v['ch9'],v['ch19'])

print(T4,OPR,'CST Fsp %.2f TSFC %.4f f %.4f Tt3 %.1f Tt45 %.1f Tt5 %.1f'%(c['Fsp'],c['TSFC_lb'],c['f'],c['Tt3'],c['Tt45'],c['Tt5']))

sizing.py (component sizing)

"""Component sizing at the SLS take-off design point (uses varcycle.py gas model)."""

import numpy as np

import varcycle as V

R = 287.0

def gamma(T, f):

c = V.cp(T, f)

return c / (c - R)

def flow_area(m, Tt, Pt, M, f=0.0):

"""Area from the compressible flow function, plus an independent rho*V*A check."""

g = gamma(Tt / (1 + 0.2 * M * M), f)

for _ in range(5):                       # iterate gamma at the static temperature

T = Tt / (1 + (g - 1) / 2 * M * M)

g = gamma(T, f)

mfp = np.sqrt(g / R) * M * (1 + (g - 1) / 2 * M * M) ** (-(g + 1) / (2 * (g - 1)))

A = m * np.sqrt(Tt) / (Pt * mfp)         # method 1: flow function

P = Pt * (T / Tt) ** (g / (g - 1))

A_check = m / ((P / (R * T)) * M * np.sqrt(g * R * T))   # method 2: rho*V*A

return A, A_check

def size_engine(st, m0, BPR, cool, f, M19, M2=0.60, htr=0.30, V_ref=25.0):

mc, mb = m0 / (1 + BPR), m0 * BPR / (1 + BPR)

out = {}

out['A2'] = flow_area(m0, st['Tt2'], st['Pt2'], M2)

out['D_fan'] = np.sqrt(4 * out['A2'][0] / (np.pi * (1 - htr ** 2)))

out['A25'] = flow_area(mc, st['Tt25'], st['Pt25'], 0.50)

out['A3'] = flow_area(mc, st['Tt3'], st['Pt3'], 0.25)

rho3 = st['Pt3'] / (R * st['Tt3'])

out['A_ref'] = (mc / (rho3 * V_ref),) * 2

out['A4'] = flow_area(mc * (1 - cool + f), st['Tt4'], st['Pt4'], 1.0, st['fb'])

out['A5'] = flow_area(mc * (1 + f), st['Tt5'], st['Pt5'], 0.45, f)

out['A8'] = flow_area(mc * (1 + f), st['Tt5'], st['Pt9'], 1.0, f)

out['A19'] = flow_area(mb, st['Tt13'], st['Pt19'], M19)

return out

calibrate.py (two-parameter calibration and sizing run)

"""Two-parameter calibration (T4 and a common efficiency offset) to published thrust and ICAO fuel flow."""

from scipy.optimize import fsolve

import varcycle as V

F_PUB  = 116988.0          # N   (26,300 lbf)

MF_PUB = 1.221             # kg/s, ICAO take-off fuel flow

M0     = 752 * 0.45359237  # kg/s, published airflow (752 lb/s)

BPR, OPR = 5.1, 27.2

BASE = dict(eta_f=0.88, eta_b=0.86, eta_c=0.87, eta_tH=0.90, eta_tL=0.91)

def residuals(x):

T4, d = x

eta = {k: v + d for k, v in BASE.items()}

r = V.cycle(T4, OPR=OPR, BPR=BPR, **eta)

return [r['Fsp'] * M0 - F_PUB,                 # thrust residual, N

r['f'] * M0 / (1 + BPR) - MF_PUB]      # fuel-flow residual, kg/s

T4, d_eta = fsolve(residuals, [1600.0, 0.03])

eta = {k: v + d_eta for k, v in BASE.items()}

r = V.cycle(T4, OPR=OPR, BPR=BPR, **eta)

print(f"T4 = {T4:.1f} K   eta offset = {d_eta:+.4f}   TSFC = {r['TSFC']:.4f} lb/(lbf.h)")

print(f"Tt3 = {r['Tt3']:.1f} K  Tt45 = {r['Tt45']:.1f} K  Tt5 = {r['Tt5']:.1f} K  f = {r['f']:.5f}")

if __name__ == "__main__":

import sizing, numpy as np

st = {k: r[k] for k in ('Tt2','Pt2','Tt13','Pt13','Tt25','Pt25','Tt3','Pt3','Tt4','Pt4','Tt5','Pt5','Pt9','Pt19','fb')}

PR = r['Pt19'] / 101325.0; g = sizing.gamma(r['Tt13'] * 0.97, 0.0)

M19 = np.sqrt(2 / (g - 1) * (PR ** ((g - 1) / g) - 1))

s = sizing.size_engine(st, M0, BPR, r['cool'], r['f'], M19)

for k, v in s.items():

print(k, np.round(v, 4))

Expected output: T4 = 1602.1 K, eta offset = +0.0387, TSFC = 0.3685 lb/(lbf.h), Tt3 = 776.2 K, Tt45 = 1193.4 K, Tt5 = 913.8 K, D_fan = 1.5367 m.

C. Abbreviations

BPR: bypass ratio. EGT: exhaust gas temperature. FPR: fan pressure ratio. HPC / HPT: high-pressure compressor / turbine. ICAO: International Civil Aviation Organization. ISA: International Standard Atmosphere. LHV: lower heating value. LPC / LPT: low-pressure compressor / turbine. NGV: nozzle guide vane. OPR: overall pressure ratio. SLS: sea-level static. TSFC: thrust specific fuel consumption. δη: common efficiency offset.